FOC MOTOR CONTROLLER

KiCAD Refresh — Design Review & Requirements Document

| Revision | Rev B — KiCAD Refresh |
| --- | --- |
| Date | 2026 |
| Application | Cat-Scale Bipedal Robot Leg Actuator |
| Topology | 3-Phase Sensorless FOC, Quasi-Direct Drive |
| Bus Voltage | 12S LiPo — 42–50.4V nominal, 60V absolute max |
| Phase Current | 10A RMS / 14A peak per phase |
| MCU | STM32F405RGT6 (Rev B) → STM32G474 (consider Rev C) |

# 1. Critical Fixes — Must Implement Before Power-On

## 1.1 MOSFET Voltage Rating

The CSD18540Q5B MOSFETs are rated 40V Vds — this is below the 50.4V fully-charged 12S bus. Inductive switching transients will spike 20–30% above bus voltage, reaching 65–75V. These devices will fail on first power-up.

| Part Number | Vds | Rds(on) | Cost (ea) | Notes |
| --- | --- | --- | --- | --- |
| CSD19537Q3 | 100V | 3.3mΩ | ~$1.50 | Direct drop-in, same package, recommended |
| BSC093N10NS3 | 100V | 9.3mΩ | ~$0.80 | Budget option, higher conduction loss |
| IPT015N10N5 | 100V | 1.5mΩ | ~$2.00 | Best performance, higher cost |

## 1.2 VCAP Capacitors (STM32F405 Internal Regulator)

C7 and C8 (VCAP_1, VCAP_2) are 100nF. The STM32F405 datasheet requires 1µF on these pins. Incorrect values cause internal LDO instability and MCU latch-up.

- Change C7, C8 to 1µF 0603 X5R — same footprint, critical fix

## 1.3 Input Transient Protection

No TVS protection exists on the motor power rail. With a 12S LiPo and inductive motor loads, transients will exceed 60V without clamping.

- Add SMBJ58A or P6KE62A TVS diode at the XT60 input — between power rail and GND

- Justifies use of 63V-rated bulk capacitors if clamping voltage is verified below 60V

# 2. Power Stage Improvements

## 2.1 Bulk Capacitance — Derived Requirements

Your existing 28× 4.7µF 1206 capacitors suffer severe DC bias derating at the 50V bus voltage. Effective capacitance at 50V is approximately 32% of nominal:

- Nominal: 28 × 4.7µF = 131.6µF

- Effective at 50V: ~42µF — well below the 175–350µF target

Target derivation for 10A/phase, 20kHz switching:

- Ripple current (RMS worst case at m=0.6): ~3.5A RMS → target 5A RMS rating with margin

- Minimum capacitance at 20kHz: C = I_peak / (f_sw × ΔV) = 7 / (20k × 1) = 350µF

- Minimum capacitance at 40kHz: 175µF

Recommended combination:

| Component | Value | Voltage | Package | Purpose |
| --- | --- | --- | --- | --- |
| Panasonic EEVTG1J101M (×2) | 100µF | 63V polymer | 8×11.5mm | Bulk energy storage |
| TDK C3225X7S2A106M (×10) | 10µF | 100V 1210 MLCC | 3.2×2.5mm | HF decoupling, ~45µF effective |
| Total effective | ~245µF | — | — | Meets 175–350µF target |

## 2.2 Input Ferrite Bead

Add a current-rated ferrite bead between the XT60 connector and the bulk capacitor bank. This prevents conducted emissions from propagating back up the power harness.

- BLM41PG221SN1 or equivalent — rated above 10A DC, low DCR

- Place immediately after XT60, before capacitor bank

## 2.3 Gate Resistor Tuning

Current R30–R41 values are 10Ω. The 1N4148 diodes for asymmetric turn-on/off are a good approach. At 20–40kHz, 10Ω may cause excessive switching losses.

- Verify with oscilloscope — check for ringing on phase nodes

- Start at 10Ω, try 4.7Ω if switching loss is high

- Keep asymmetric turn-on (via diode) to control dI/dt independently from turn-off

# 3. Sensing & Control Architecture

## 3.1 Phase Voltage Sensing — Architecture Clarification

The existing sensing circuit (39k/2.2k divider, 100nF cap, 3.3V zener) measures phase voltage relative to power ground. The divider ratio is correct (3.2V at 60V). However, for sensorless FOC the role of this circuit is frequently misunderstood.

Two distinct approaches exist — only one is appropriate for FOC:

|  | Trapezoidal / 6-Step | Sensorless FOC (Your Target) |
| --- | --- | --- |
| Method | Back-EMF zero crossing | State observer (SMO, EKF, Luenberger) |
| Phase sensing | Required — floating phase zero cross | Not required — observer uses reconstructed voltage |
| Virtual star point | Used — 3× equal R to neutral | Not applicable — no floating phase exists |
| Observer inputs | Phase voltages directly | Phase currents + duty cycle × Vbus |

Conclusion: Keep the VPHASE_A/B/C sensing circuit for diagnostics and observer cross-checking at speed. Do not rely on it as the primary angle estimation source. The PWR_VSENS bus voltage measurement is essential and is already correctly implemented.

## 3.2 Phase Voltage Filter Correction

Current RC filter: 2.2kΩ / 100nF → fc = 723Hz. This is likely too low and will attenuate the signal at medium-high electrical frequencies, with significant phase shift.

- Determine max electrical frequency: f_elec = (RPM/60) × pole_pairs

- U8 II has 21 pole pairs — at 640 RPM motor: f_elec = 10.67 × 21 = 224Hz → filter is adequate

- At higher motor speeds, revisit — consider 4.7kΩ / 10nF → fc ≈ 3.4kHz

- ADC sampling MUST be synchronized to PWM center (center-aligned timer trigger) — not free-running

- Implement dead-time compensation in firmware, not in the analog domain

## 3.3 Current Sensing

Shunt resistor spec (5mΩ, 3W, ±1%, ±50ppm/°C) is correct for 20A peak headroom with 100mV sense voltage. The DRV8323 integrated current sense amplifiers are used correctly.

- Verify SOA/B/C gain setting via R24–R26 (56Ω currently) during hardware bring-up

- Add CAL routine in firmware triggered over CAN/UART for zero-current offset calibration

- Add test points on ISENSA/B/C after shunt resistors — essential for calibration

# 4. EMC — Good Practice Without Lab Access

## 4.1 Ground Architecture

The most impactful EMC decision made at layout time. Three separate grounds must be maintained and joined at one star point:

| Ground | Net Name | High dI/dt? | Star Point Location |
| --- | --- | --- | --- |
| PGND | DRV8323 PGND (pin 40) | Yes — switching return | Near shunt resistors |
| AGND | DRV8323 AGND (pin 32) | No — analog sense | Near shunt resistors |
| DGND | MCU VSS, logic ICs | Low — digital switching | Near decoupling caps |

- Never route digital signals over the power stage ground plane

- Full copper pour on the layer directly beneath all switching nodes

- Join all three grounds at one point only — near the shunt resistors

## 4.2 Decoupling Placement Rules

- All decoupling caps on the same layer as the IC they serve

- Within 0.5mm of the supply pin — via directly to ground plane

- DRV8323 bootstrap caps (C19, CPH/CPL) must have minimum loop to PGND

- Gate resistors (R30–R41) placed directly at MOSFET gate pins — not routed away

## 4.3 CAN Bus EMC

- Add common mode choke on CAN_H/CAN_L before connector — DLW21SN900SQ2 or equivalent (~$0.30)

- Place termination resistor R62 and CM choke at the connector, not near the transceiver

- Use twisted pair for CAN harness — 24 AWG twisted, 120Ω characteristic impedance

# 5. Design for Test (DFT)

## 5.1 Test Point Coverage

Current TPs are not systematic. Required additions:

| Category | Signals | Status | Action |
| --- | --- | --- | --- |
| Power Rails | MOTOR_POWER, VDRAIN, 5V_SEL, 3V3_LDO | Partial | Add near each source |
| Phase Outputs | VPHASE_A, VPHASE_B, VPHASE_C | Missing | Add — highest priority |
| Current Sense | ISENSA, ISENSB, ISENSC after shunts | Missing | Add for calibration |
| Sense Amplifier | SOA, SOB, SOC outputs from DRV8323 | Existing (TP9/14/15) | Keep |
| GND Returns | GND TPs every 2–3cm across board | Missing | Critical for probing |

## 5.2 SWD Interface Upgrade

- Replace current 2.54mm header P1 with Tag-Connect TC2030-CTX-NL footprint

- No connector on board — programmer clips on for programming, no stub when deployed

- Add 10-pin 1.27mm IDC footprint (ARM Cortex standard) for development boards — leave unpopulated in production

- Connect SWO (PA14/TRACESWO) to TC2030 pin 6 — enables ITM trace logging

## 5.3 Bring-Up Test Firmware

Write a self-test routine that runs on first boot and reports pass/fail over UART/CAN:

- Sweep PWM 0→100% on each phase, monitor SOA/B/C via ADC

- Report all ADC rail voltages over UART on boot

- Blink D2/D6 green LEDs in pass/fail pattern

- Zero-current offset calibration triggered by CAL pin or CAN command

# 6. Design for Manufacturing (DFM)

## 6.1 Component Rationalization

Audit BOM for duplicate values across different packages. Every unique line item is a potential stock-out and an extended part fee at assembly houses.

- Standardize decoupling caps: 100nF → 0402 X5R throughout logic section

- Consolidate 10kΩ pull-ups to single package (0402 or 0603 — pick one)

- Check all parts against JLCPCB/PCBWay basic library — extended parts add ~$3/unique part

- Target: reduce unique BOM line items by 20–30%

## 6.2 Fiducial Markers

- Add 3 fiducial markers on copper layer — asymmetric triangle arrangement

- 1mm copper circle, 3mm soldermask opening — required for optical pick-and-place

- Place on component side, away from board edge

## 6.3 Silkscreen Standards

- Every component: visible reference designator on assembled side

- Pin 1 dot on all ICs

- Polarity bar on all diodes and polarized capacitors

- Board revision, date code, project name on silkscreen

- Every connector: function label + pin 1 + pinout on silkscreen

- Consider QR code linking to GitHub/portfolio — unusual and memorable

## 6.4 PCB Stack-Up

- 2oz copper on outer layers — required for 3.5mm minimum phase trace width at 10A

- 1oz copper inner layers — standard

- 1.6mm board thickness — standard, sufficient

- HASL or ENIG finish — ENIG preferred for fine-pitch ICs and long-term reliability

# 7. Communication & Connector Strategy

## 7.1 Communication Tiers

| Tier | Protocol | Hardware | Latency | Purpose |
| --- | --- | --- | --- | --- |
| 1 — RT | CAN / CAN-FD | TCAN1042 → upgrade TCAN1462 | <1ms | Joint setpoints, torque commands |
| 2 — Supervisory | USB CDC / UART | Existing USB-C + USART3 | 1–100ms | Telemetry, logging, config |
| 3 — Debug | SWD + SWO | Tag-Connect TC2030 | Non-RT | Firmware flash, GDB, ITM trace |

## 7.2 CAN Upgrade — CAN-FD

The current TCAN1042DRQ1 supports only classical CAN 2.0B (8-byte payload, 1Mbit/s). Upgrade to CAN-FD for a robot with multiple joints:

- Replace with TCAN1462 — CAN-FD, 8Mbit/s data phase, compatible pinout family, ~$1.20

- Note: STM32F405 does not have CAN-FD hardware — this becomes a stronger argument for STM32G474 in Rev C

- CAN-FD allows 64-byte payload: position + velocity + torque + status in one frame per joint at 1kHz

## 7.3 Add RS-485 Transceiver

Add a MAX3485 or SN65HVD1781 on the existing USART3 lines with a direction control GPIO. This unlocks:

- Dynamixel servo ecosystem — large library of COTS servos, grippers, sensors

- Multi-drop half-duplex at up to 10Mbit/s

- Cost: ~$0.50 for the IC, one GPIO for direction control

## 7.4 Connector Manifest

| Connector | Standard | Purpose | Qty | Action |
| --- | --- | --- | --- | --- |
| XT60 (existing) | XT60 | Main power input | 1 | Keep — add reverse polarity protection |
| XT30PW-M (add) | XT30 | Motor phase output | 1 (3-pin MR30) | Add — field-swappable motor |
| JST-GH 4-pin (add) | DroneCAN / UAVCAN | CAN bus daisy-chain | 2 | Add — robot ecosystem standard |
| JST-GH 6-pin (existing) | Pixhawk UART | Debug UART | 1 | Keep (J2/J3) |
| JST-SH 4-pin 1.0mm (add) | Qwiic / STEMMA QT | I2C expansion | 2 | Add — 100+ COTS sensors |
| Tag-Connect TC2030-CTX-NL (add) | ARM Cortex SWD | Programming / debug | 1 | Replace current header |
| USB-C (existing) | USB 2.0 | DFU + CDC UART | 1 | Keep — already implemented |
| 3.5mm screw terminal (add) | RS-485 | Dynamixel / RS-485 bus | 1 | Add with RS-485 transceiver |

## 7.5 Debugger Selection

| Debugger | Cost | Key Feature | Recommendation |
| --- | --- | --- | --- |
| ST-Link V3 MINIE | ~$10 | Virtual COM built-in | PRIMARY — no licensing concerns, STM32 native, virtual COM port = no USB-UART adapter needed |
| J-Link EDU Mini | ~$18 | RTT logging, Ozone | UPGRADE — industry standard, RTT = printf debug at zero CPU overhead. EDU license ok for portfolio. |
| Black Magic Probe | ~$60 | GDB server on probe | Open source, GDB direct connection, no driver install. Advanced users. |

# 8. Thermal Design

## 8.1 Power Dissipation Budget

| Source | At 10A RMS | Worst Case | Notes |
| --- | --- | --- | --- |
| 6× MOSFETs (Rds×I²) | ~1.8W | ~2.5W | Assuming 3mΩ Rds(on) replacement FETs |
| 3× Shunt resistors (5mΩ) | ~1.5W | ~4.0W (14A peak) | P = 3 × I² × 0.005 |
| Gate driver (DRV8323) | ~0.5W | ~1.0W | Quiescent + switching losses |
| Total PCB dissipation | ~4–5W | ~7–8W | Does not include motor winding loss |

## 8.2 Layout Requirements

- Exposed MOSFET thermal pads with via-in-pad (filled/capped) to back copper — primary heat path

- Minimum 200mm² copper pour on back side beneath MOSFET footprints tied to GND/PGND

- 2oz copper on both outer layers — reduces thermal resistance and trace impedance

- Define a flat area on PCB back for chassis contact or heatsink attachment

- Mount PCB remotely from motor for Rev B — motor vibration + heat should not reach the PCB directly

# 9. PCB Mechanical & Layout Constraints

## 9.1 Derived from Motor Selection

Motor selection: T-Motor U8 II KV100 (hip/knee, 212g, 63mm OD) and T-Motor MN5008 KV170 (abduction, 130g, 50mm OD) with 6:1 single-stage planetary gearbox.

| Parameter | Constraint | Derivation |
| --- | --- | --- |
| Board dimensions | ≤ 80 × 80mm | Full BOM requires ~70×60mm minimum |
| Board thickness | 1.6mm | Standard — sufficient for 2oz copper |
| Copper weight | 2oz outer / 1oz inner | Required for thermal and current density |
| Phase trace width (2oz) | ≥ 3.5mm | IPC-2221 at 10A, 10°C rise, 2oz Cu |
| MOTOR_POWER trace | ≥ 3.0mm | 10A continuous with thermal margin |
| Mounting holes | M3 × 5 (existing) | Publish exact positions from datum corner |
| MOSFET thermal pad area | ≥ 200mm² back copper | θ_ja ≤ 11°C/W required at 8W dissipation |
| Board mass target | ≤ 50g | Weight budget: 3.5kg total robot, 1.0kg per leg |
| Max operating temperature | 85°C | Enclosed robot body, 60°C ambient assumed |
| Vibration rating | 10G at 200Hz minimum | Motor cogging frequency with 6-pole pair minimum |

## 9.2 Reverse Polarity Protection

XT60 connectors can be physically forced in reverse polarity. Add input protection:

- LTC4359 ideal diode IC — low dropout, suitable for 60V, ~$2.50

- OR: SI7157DP P-channel MOSFET ideal diode — simpler, ~$0.50 + gate resistor + zener

- Place immediately after XT60, before ferrite bead and bulk caps

# 10. What to Keep from Rev A

The following elements are correctly implemented and should be carried forward unchanged or with minor refinements:

| Element | Why It’s Correct |
| --- | --- |
| DRV8323HRTAR gate driver | 60V-rated, integrated current sense amps, SPI config, nFAULT reporting. Excellent choice. |
| HW config via 0Ω resistors (MODE, GAIN, IDRIVE, VDS) | Enables rework without PCB changes. Correct approach for first revision. |
| Shunt resistor spec (5mΩ, 3W, ±1%, ±50ppm/°C) | Correct for 20A headroom, 100mV sense voltage, low tempco for FOC accuracy. |
| Phase voltage zener clamp (3.3V) | Protects MCU ADC input correctly. Keep — verify zener placement relative to filter. |
| PWR_VSENS bus voltage sensing | Essential for FOC voltage reconstruction. Correctly implemented. |
| 1N4148 diode on gate resistors | Asymmetric turn-on/off speed control. Good practice for dV/dt management. |
| AS5600L hall encoder with I2C2 | Good choice for absolute position. Fixed I2C address 0x36 — note limitation for multi-encoder expansion. |
| CAN transceiver + switchable 120Ω terminator | SW3 termination switch is thoughtful for multi-node robot systems. |
| USB-C with CC resistors + OR-ing diode | Correctly implemented. Keep for DFU field updates and CDC virtual COM. |
| TMP235 analog temperature sensor | Simple, linear output, feeds MCU ADC directly. Sufficient for thermal protection. |
| ESD protection (PACDN042Y3R) on USB and UART | Good practice, correctly placed. |
| BOOT0 / SWD debug via SW1/SW2 | Rework-friendly. Keep — replace physical header with Tag-Connect footprint. |
| 60V to 5V buck (LMR51610XFDBVR) | Appropriate for 65V input range. Keep feedback resistor values. |
| 5V to 3V3 LDO (NCP176AMX330TCG) | 500mA LDO with enable pin. Correctly implemented. |
| 5 M3 mounting holes | Good coverage. Add exact coordinates to mechanical drawing. |

# 11. MCU Migration Path — Rev B vs Rev C

The STM32F405 is functional but is not optimized for motor control. Evaluate migration based on revision scope:

|  | STM32F405 (Rev B — Keep) | STM32G474 (Rev C — Target) |
| --- | --- | --- |
| Cost | ~$8–12 | ~$3–5 |
| FOC Timer | TIM1 — standard, adequate | HRTIM — 184MHz resolution, superior PWM quality |
| Math Accelerators | None — software only | CORDIC (trig) + FMAC (filters) in hardware |
| Op-Amps | None on-chip | 3× on-chip op-amps — can simplify current sense |
| CAN | bxCAN — classical only | FDCAN — CAN-FD hardware support |
| Package | LQFP64 | LQFP64 or QFN32 option |
| Recommendation | Complete Rev B with F405 to validate hardware | Migrate to G474 in Rev C after control loop is validated |

# 12. Motor Selection & System Architecture

## 12.1 Recommended Motors (Cat-Scale Bipedal Leg)

| Joint | Motor | Gear Ratio | Output Torque | Notes |
| --- | --- | --- | --- | --- |
| Hip flex/ext | T-Motor U8 II KV100 (212g) | 6:1 planetary | ~8 Nm peak | Primary load bearing |
| Knee flex/ext | T-Motor U8 II KV100 (212g) | 6:1 planetary | ~8 Nm peak | Primary load bearing |
| Hip abduction | T-Motor MN5008 KV170 (130g) | 6:1 planetary | ~5 Nm peak | Lateral stability |

- Architecture: Quasi-direct drive — low reflected inertia, backdrivable, absorbs impacts

- Your 10A/phase board spec aligns well with U8 II operating point at 44.4V nominal

- PCB mounts remotely from motor for Rev B — not co-axial

## 12.2 Harness Specifications

| Harness | Wire Gauge | Max Length | Notes |
| --- | --- | --- | --- |
| Motor phase wires | 18 AWG minimum, 16 AWG preferred | ≤ 200mm | Twist all 3 phases together — reduces radiated EMI |
| Power supply harness | 12 AWG minimum | ≤ 300mm | Twist power pair — each 100mm adds ~100nH inductance |
| CAN bus harness | 24 AWG twisted pair | Network dependent | 120Ω characteristic impedance |

# 13. Rev B Implementation Priority Checklist

Use this checklist when starting the KiCAD refresh:

| P | Action Item | Section Ref |
| --- | --- | --- |
| P1 | Replace CSD18540Q5B with 100V-rated MOSFETs (CSD19537Q3 recommended) | Section 1.1 |
| P1 | Change C7, C8 (VCAP_1/2) from 100nF to 1µF | Section 1.2 |
| P1 | Add TVS diode (SMBJ58A) across power input rail at XT60 | Section 1.3 |
| P2 | Add 2× 100µF 63V polymer + 10× 10µF 100V 1210 MLCC bulk capacitors | Section 2.1 |
| P2 | Add input ferrite bead between XT60 and bulk cap bank | Section 2.2 |
| P2 | Add reverse polarity protection (LTC4359 or SI7157DP) at power input | Section 9.2 |
| P2 | Add CAN-FD transceiver TCAN1462 to replace TCAN1042 | Section 7.2 |
| P2 | Add MR30 / XT30 connector for motor phase output | Section 7.4 |
| P3 | Replace SWD header P1 with Tag-Connect TC2030-CTX-NL footprint | Section 5.2 |
| P3 | Add systematic test points: VPHASE_A/B/C, ISENSA/B/C post-shunt, GND returns | Section 5.1 |
| P3 | Add common mode choke on CAN_H/CAN_L before connector | Section 4.3 |
| P3 | Add RS-485 transceiver (MAX3485) on USART3 with DIR GPIO | Section 7.3 |
| P3 | Add 2× JST-SH 4-pin 1.0mm Qwiic connectors on I2C2 bus | Section 7.4 |
| P3 | Add 2× JST-GH 4-pin DroneCAN connectors for CAN daisy-chain | Section 7.4 |
| P4 | Rationalize BOM: standardize 100nF to 0402, all 10k pull-ups to one package | Section 6.1 |
| P4 | Add 3 fiducial markers in asymmetric triangle arrangement | Section 6.2 |
| P4 | Update silkscreen: all connectors labelled with function and pinout | Section 6.3 |
| P4 | Verify and correct phase voltage filter cutoff for U8 II motor electrical frequency | Section 3.2 |
| P4 | Expose MOSFET thermal pads with via-in-pad, add back copper pour ≥200mm² | Section 8.2 |
| P4 | Publish mechanical drawing: board outline, mounting holes, connector positions | Section 6 / 9 |

FOC Motor Controller — Rev B Design Review    |    For KiCAD Refresh